Computer Networks: ITEC611 (Troubleshooting)

Full Name: Amir Tamang	                                                                                                                                     Student Number: S00393664

Part A:

| No. | Ticket Desc. | Problem | Show Command Used | Command to fix the problem |
| --- | --- | --- | --- | --- |
| Example | R1 cannot ping VLAN 5 on MGT-SW | The IP address & subnet mask are not configured correctly | MGT-SW# Show ip interface brief MGT-SW# Show interface vlan 5 | MGT-SW# configure terminal MGT-SW (config)# int vlan 5 MGT-SW(if-config)# ip add 5.5.5.10 255.255.255.0 |
| 1. | SW0 cannot ping Vlan 1 on SW2 | VLAN 1 is not configured on SW2 (1 mark) | SW2# show ip interface brief SW2# show interface vlan 1 (1 mark) | SW2# configure terminal SW2(config)# interface vlan 1 SW2(if-config)# ip address 192.168.1.2 255.255.255.0 SW2(if-config)# no shutdown (2 marks) |
| 2. | R3 is unable to get ping response from R1’s Loopback 0. | Routing or IP configuration issue (1 mark) | R1# show ip interface brief R3# show ip route (1 mark) | R1# configure terminal R1(config)# interface loopback 0 R1(if-config)# ip address 192.168.7.1 255.255.255.0 R1(if-config)# no shutdown (2 marks) |
| 3. | SW1 is not reachable from R3. | VLAN or routing configuration issue (1 mark) | SW1# show ip interface brief R3# show ip route (1 mark) | SW1# configure terminal SW1(config)# interface vlan 1 SW1(if-config)# ip address 192.168.2.125 255.255.255.0 SW1(if-config)# no shutdown (2 marks) |
| 4. | Identify the IOS version, NVRAM and ROM size for R2 router | IOS version: Version 15.1(4) M4 (1 mark) | Show command: R2# Show version (1mark) | NVRAM: 255K bytes ROM: System Bootstrap (2 marks) |

Part B:

- Identify the route (IP addresses) a packet takes to go from SW0 to Lo1 on R3. What command (write the full command) did you use for the above?  (2 marks)

- SW0# traceroute 200.200.200.1

- Calculate the average round trip time (RTT) for the following route: SW0 ->SW2. What command (full command) did you use to find the RTT value? (2 marks)

- The code I used to find the RTT value is:

SW0# ping 192.168.1.2